## 5.13. Sheet NM, Net Metering Customers

The table should contain information on all NM customers within the DU’s franchise area.

The input fields are as follows:

| Input Field | Description |
| --- | --- |
| Customer Name | Name of the customer (or customer number if there are concerns on data privacy) |
| RE Resource | Constrained to “Solar”, “Wind”, “Biomass or Biogas”, and “Other RE Resource” |
| Installed kW | Installed capacity in kW |
| Import (M1) | Energy in kWh imported or drawn from the Distribution System for the previous year |
| Export (M2) | Energy in kWh exported or injected to the Distribution System for the previous year |
| Gross (M3) | Generated kWh |

The entries should not be separated by empty rows.

The values for Gross (M3) must come from REC meters as required under Section 10 of the “Amended Rules Enabling the Net Metering Program for Renewable Energy” adopted under ERC Resolution No. 06, Series of 2019.

Populate the input fields as applicable. To check for correctness, save the file then hit . To clear the contents of the table, click the Clear button.

Clicking  for Sheet NM yields a summary window if there are no errors, or an error message as follows:

| Input Field | Condition |
| --- | --- |
| RE Resource | Not in the drop-down list |
| Installed kW | Text value |
| Import (M1) | Text value |
| Export (M2) | Text value |
| Gross (M3) | Text value |
| Export (M2), Gross (M3) | Export (M2) > Gross (M3) |

If the customer counts in NM and the last historical year in Cust are not the same, the summary chart will indicate “Discrepancy”.