## 2.2. Sheet Hist, Historical Consumption Data

The table should contain the historical MW and MWh consumption of the Sector.

Annual historical data is required from 2000 to 2015. Monthly historical data is required from 2016 onwards. Clicking the up spinner button will add twelve columns, one for each month of the following year. This is useful for using the same file for future DDP submissions. Clicking the down spinner button will delete the twelve columns for the months of the last historical year unless it is 2018. This cannot be undone.

The input fields are as follows:

| Input Field | Description |
| --- | --- |
| Coincident Peak MW | Coincident peak or system peak in MW for the Sector for the year or month, with generation from NM customers and SGF taken into account |
| MWh Offtake | Total energy in MWh purchased from Generators, WESM, and other DUs through Sale for Resale Agreements, plus the total energy in MWh generated by RES for the DU’s switched contestable Customers, plus the total energy in MWh injected by NM Customers, SGFs (generation from embedded generators not covered by a PSA may be lumped with SGF), DU-owned plants, and embedded plants for the Sector for the year or month |
| WESM | Energy in MWh purchased from WESM for the Sector for the year or month (may be negative if the quantities sold to the spot market exceed the quantities purchased) |
| MWh Input | MWh Offtake less Transmission Loss for the Sector for the year or month |
| [MWh Sales (Old)] = Residential, Commercial, Industrial, Public Buildings, Streetlights, Water Systems, All Others | MWh Sales for each customer class for the Sector for the year or month |
| [MWh Sales (New)] = Residential, Low Voltage, Higher Voltage | MWh Sales for each customer class for the Sector for the year or month based on a different classification |
| S4R as Seller | Energy Sales in MWh to other DUs through Sale for Resale Agreements for the Sector for the year or month (to be populated only by the Selling DU) |
| Own Use | Own Use in MWh for the Sector for the year or month |
| Switched Contestable Customers | Energy in MWh delivered to Contestable Customers served by RES |
| Feeder Technical Loss | Losses in MWh incurred by the distribution network downstream of the distribution substations for the Sector for the year or month |
| SubTx & SS Technical Loss | Losses in MWh incurred by the sub-transmission network and the distribution substations for the Sector for the year or month |
| Non-technical Loss | Non-technical component of System Loss in MWh for the Sector for the year or month |

Before populating the table, click the Refresh button to populate MWh Offtake with the sum of the entries in Offtake, and WESM with the contents of the second row of Offtake, for the last historical year. The rest of the cells of MWh Offtake and WESM will have to be filled in manually.

For Grid Sectors, MWh Input shall be the quantity measured at the metering points, which is expected to deviate from MWh Offtake, which is basically the total purchased quantity. Transmission Loss accounts for this difference. For SIIG and S4R Sectors, it is expected that MWh Offtake is equal to MWh Input as there is no valid Transmission Loss.

The Buying DU shall have a PSPP for a Sector served by the Selling DU under a Sale for Resale Agreement. In said PSPP, the Buying DU shall populate MWh Offtake and MWh Input according to the energy sold by the selling DU and populate MWh Sales (Old) and MWh Sales (New) accordingly. S4R as Seller must be left blank.

The Selling DU shall indicate in its PSPP for “Grid” the energy sold to the Buying DU in S4R as Seller and not in MWh Sales (Old) and MWh Sales (New).

The DU’s consumption should be in Own Use and not in Commercial.

As required by the “Methodology for Segregating DSL” under the “Rules for Setting the Distribution System Loss Cap and Establishing Performance Incentive Scheme for Distribution Efficiency” adopted under ERC Resolution No. 20, Series of 2017, Feeder Technical Loss and SubTx & SS Technical Loss shall be determined based on Load Flow Analysis. However, if there is no System Loss segregation in place, input the entire quantity to Feeder Technical Loss. Likewise, input the entire quantity to Feeder Technical Loss if the technical component of System Loss is known but cannot be segregated further. For such cases, include the necessary narrative to the Draft PSPP.

Any imbalance between MWh Input and the sum of MWh Sales (Old), S4R as Seller, Own Use, Switched Contestable Customers, Feeder Technical Loss, SubTx & SS Technical Loss, and Non-technical Loss will be reflected as a non-zero value in MWh Discrepancy below the table. The corresponding cell will be colored blue (input excess or output shortfall) or red (input shortfall or output excess). Don’t manipulate the data just to force MWh Discrepancy to a value of zero.

Any imbalance between MWh Offtake and MWh Input will be reflected as a non-zero value in MWh (Input – Offtake) below the table. The corresponding cell will be colored blue (offtake excess or input shortfall) or red (offtake shortfall or input excess). The same imbalance will be used to calculate Transmission Loss. For a correct set of data, Transmission Loss will not have a negative value. As a matter of fact, it should be positive for most Grid-connected Sectors as MWh Offtake is naturally higher than MWh Input.

Any imbalance between MWh Sales (Old) and MWh Sales (New) will be reflected as a non-zero value in MWh Sales (New vs Old) below the table. The corresponding cell will be colored blue (“Old” excess or “New” shortfall) or red (“Old” shortfall or “New” excess). Regardless of any conflict, MWh Sales (Old) will be the one used in calculations.

Populate the input fields as applicable. To check for correctness, save the file then hit . To clear the contents of the table, click the Clear button.

Clicking  for Sheet Hist yields a summary window if there are no errors, or an error message if there is a text value where none should be.