## 2.3. Sheet LP, Load Profile for Previous Year

The table should contain the instantaneous MW for each metering point in the Sector.

Clicking the up spinner button will add a column for a metering point. Clicking the down spinner button will delete the last column unless only one remains. This cannot be undone.

The input fields are as follows:

| Input Field | Description |
| --- | --- |
| Metering Point | Name of the metering point (or substation) where load profile data was measured |
| Substation MVA | Substation capacity in MVA if Metering Point is located in a DU substation, or the load-carrying capacity in MVA of the DU's subtransmission line that tapped to an upstream transmission line, as applicable. |
| DateTime | The date and time a measurement was made, with the constraint that the sampling interval must be constant |
| MW | The instantaneous load in MW at the substation at the date and time of measurement |

Metering Point must be filled with unique values as no two metering points are the same.

The inputs to DateTime and MW must come from a reputable data logging system. Otherwise, there may be a need to pre-process the data outside of the PSPP template to follow the format.

A constant sampling interval of one hour is good enough. A sampling interval of less than five minutes will slow down the processing too much. Be careful in manipulating data as non-uniform increments can be difficult to detect and can lead to failed charting of the load curve.

For Sale for Resale Sectors, it is possible that data logging equipment are not in place at the tapping point. In such a case, just leave the table as it is but include a narrative to the Draft PSPP.

If meter data provided by the National Grid Corporation of the Philippines (NGCP) is in kW, divide by 1,000 before pasting as values to the table. It was generally observed that using the same dataset from NGCP, the values for DateTime may be obtained by adding the date and time in a separate column and pasting as values into DateTime. Do not use the Concatenate function as it yields text values.

Populate the input fields as applicable. To check for correctness, save the file then hit . To clear the contents of the table, click the Clear button.

Clicking  for Sheet LP yields a summary window if there are no errors, or an error message as follows:

| Input Field | Condition |
| --- | --- |
| Metering Point | Duplicate values |
| Substation MVA | Text value |
| DateTime | Text value or blank |
| MW | Text value |